Supporting Print Devices

CompTIA A+ 220-1201 | Module 10 – Student Study Guide

Episode Title: The Machines That Never Sleep

Learning Objectives

By the end of this lesson, students will be able to:

- Identify major printer types and their use cases

- Explain the seven-step laser printing process in order

- Compare laser, inkjet, thermal, and impact printers

- Recognize common printer components and failures

- Understand printer connectivity and networking

- Describe printer security risks and protections

- Apply real-world troubleshooting logic for printer problems

Key Vocabulary (Tested on A+)

- PDL (Page Description Language) – Language used by printers to process jobs

- PCL – Fast, universal printer language

- PostScript – High-quality graphics language

- CMYK – Cyan, Magenta, Yellow, Black color model

- Drum – Photosensitive imaging cylinder

- Fuser – Bonds toner using heat and pressure

- Pickup Roller – Feeds paper into printer

- Print Spooler – Manages print job queues

- Secure Print – Requires authentication before release

- ADF – Automatic Document Feeder

- OCR – Optical Character Recognition

Printer Types & Uses

Laser Printers

- Business & schools

- High speed, high volume

- Uses toner

Inkjet Printers

- Home & photos

- High color quality

- Uses liquid ink

Thermal Printers

- Receipts & labels

- Heat-sensitive paper

- No ink or toner

Impact (Dot Matrix) Printers

- Warehouses & shipping

- Multi-part forms

- Loud, low quality

The 7-Step Laser Printing Process (MEMORIZE)

- Processing – Job converted to bitmap

- Charging – Drum given uniform negative charge

- Exposing – Laser removes charge to form image

- Developing – Toner sticks to exposed areas

- Transferring – Paper pulls toner from drum

- Fusing – Heat & pressure permanently bond toner

- Cleaning – Excess toner removed

Exam Clues:

- Toner smears → Fuser failure

- Repeating defects → Drum issue

- Light prints → Low toner

Printer Connectivity

- USB – One computer only

- Ethernet – Network printer with IP

- Wi-Fi – Wireless printing

- Bluetooth – Short-range mobile

Printer Sharing Models

- Local shared printer

- Print server (enterprise standard)

- Public printer

If multiple printers fail at once → suspect the print server

Common Printer Failures & Causes

| Issue | Likely Cause |
| --- | --- |
| Toner wipes off | Failed fuser |
| Vertical lines | Damaged drum |
| Blank pages | Empty or sealed toner |
| Paper jams | Worn rollers or humidity |
| Printer offline | IP or port changed |
| Wrong colors | CMYK imbalance |

Printer Security Essentials

- Printers contain hard drives, RAM, and network cards

- Use secure print with PIN or badge

- Perform secure erase before disposal

- Keep on separate VLAN

- Update firmware regularly

Unsecured printers cause silent data breaches

Mobile & Cloud Printing

- Apple AirPrint

- Android Mopria

- Manufacturer mobile apps

- Cloud print services

Security Risks:

- Open discovery

- Unauthenticated release
 Always enforce authentication

Environmental Threats

- Humidity → jams

- Dust → roller slippage

- Cheap toner → fuser damage

- Cold rooms → condensation

Exam Strategy Tips (Prof JRod)

- Toner smears → FUSER

- Multiple printers fail → PRINT SERVER

- Colors wrong → CMYK TONER

- Printer disappears → IP CHANGE

- Documents release freely → NO SECURE PRINT

Quick Knowledge Check

- Which component uses heat and pressure?

- Which printer uses heat-sensitive paper?

- What causes repeating print defects?

- What service controls the queue?

- What security feature prevents instant release?